# DTUsat Power Summary

Last edited March 16, 2003

Most recent version is found at:

DTUsat -> SysEng -> Files -> Systembeskrivelser -> power.doc

## System Function

The Power system’s main function is to deliver a stable 3V6 DC voltage to the other systems. The Power system must be able to handle a latch up in some of the other systems. When a latch up occur, the power system will shut down the satellite for a while and then turn it back on.

During latch up shut down everything but the latch-up-free latch up system will be turned off. This includes the DC-DC converter, Battery protection circuitry and the house keeping data measurement circuitry in the power system.

NOTICE: The DC-DC converter and the Battery protection circuitry in the power system is not latch up protected, which means that they have no latch up detection.

This means that if a latch up only occur in the DC-DC converter and the Battery protection circuitry the hole satellite will be lost. But if a latch up occur at the same time in one of the other latch up protected circuits the satellite will survive.

In all probability the latch up- storm will generate more than one latch up at a time.

It is only destructive latch up that can be detected, micro latch up that causes systems to stop function can’t be detected. But these micro latch up`s will only be removed after a shut down caused be a destructive latch up.

## Mass

Power PCB:		 42 g 	including OBC pins

Harness: 	                max  5 g 	Solar and battery cables.

Battery:		 42 g 	Battery attachment is not included.

## Physical Dimensions

55 mm x 85 mm

## Contact Information

Thomas Jeppesen

Bergsøekollegiet 1208

2850 Nærum

E-mail: c971607@student.dtu.dk

Phone: 77 42 51 40

Mobil: 28 46 27 25

## Block Diagram of Power system

Not ready

## Picture of Power system

Not ready

## Connections to OBC

| Pad | Connection |
| --- | --- |
| 1+2 | PwrSolar |
| 3+4 | Bypass latch up system |
| 5+6 | Flight pin |
| 7+8 | VBat |
| 9+10 | VUnreg |
| 11+12 | Gnd |
| 13+14 | 3V6 |
| 15+16 | Latch up |
| 17+18 | Not used |
| 19 | SPI_SEL3 |
| 20 | SPI_SEL4 |
| 21 | SPI_OUT |
| 22 | SPI_IN |
| 23 | SPI_CLK |
| 24 | Sign (oplad/aflad) |
| 25 | Temp |
| 27+28 | 3V3 REG |
| 29+30 | 3V6 |
| 31+32 | Gnd |
| 33+34 | VUnreg |

## House Keeping Data

# Voltage Measurement

|  | Scaling Factor | Max series resistance | Resistors (R1:R2) | Actual Scale | VREF |
| --- | --- | --- | --- | --- | --- |
| VUnreg | 1:3 | 1.3MΩ | 430K : 820K | 0.344 | 3V3 |
| VBat | 1:2 | 1.3MΩ | 620K : 620K | 0.5 | 3V3 |
| 3V6 | 3:5 | 1.8MΩ | 1M : 680K | 0.595 | 3V3 |

Vmeas o----/\/\/\/----o----/\/\/\/----o Gnd

R2            |        R1

o adc

## Current Measurement

## Not ready

# Sign Measurement

## Not ready

# Temperature Measurement

## Not ready

## Current Project State

Her Mangler status (Remaining Work ikke helt gyldig), kommer snart!

Remaining Work:

- Assembly of Flight and Flight spare PCB

- Integration Tests